### LED Array Effect use For Loop Iteration

Often you want to iterate over a series of pins and do something to each one. For instance, this example blinks 6 LEDsattached to the Arduino or Genuino by using a for() loop to cycle back and forth through digital pins 2-7. The LEDS are turned on and off, in sequence, by using both the digitalWrite() and delay() functions .

We also call this example "Knight Rider".

### Hardware Required

- Arduino or Genuino Board

- 6 220 ohm resistors

- 6 LEDs

- hook-up wires

- breadboard

### Circuit

Connect six LEDS, with 220 ohm resistors in series, to digital pins 2-7 on your Arduino.

### Schematic:

### Code

The code below begins by utilizing a for() loop to assign digital pins 2-7 as outputs for the 6 LEDs used.

In the main loop of the code, two for() loops are used to loop incrementally, stepping through the LEDs, one by one, from pin 2 to pin seven. Once pin 7 is lit, the process reverses, stepping back down through each LED.

int timer = 100;           // The higher the number, the slower the timing.

void setup() {
  // use a for loop to initialize each pin as an output:
  for (int thisPin = 2; thisPin < 8; thisPin++) {
    pinMode(thisPin, OUTPUT);
  }
}

void loop() {
  // loop from the lowest pin to the highest:
  for (int thisPin = 2; thisPin < 8; thisPin++) {
    // turn the pin on:
    digitalWrite(thisPin, HIGH);
    delay(timer);
    // turn the pin off:
    digitalWrite(thisPin, LOW);
  }

  // loop from the highest pin to the lowest:
  for (int thisPin = 7; thisPin >= 2; thisPin--) {
    // turn the pin on:
    digitalWrite(thisPin, HIGH);
    delay(timer);
    // turn the pin off:
    digitalWrite(thisPin, LOW);
  }
}

| Your full name | Tel. | Email | Group | Su23 |
| --- | --- | --- | --- | --- |
| Nguyễn Xuân Trung – qe170172 | 0944353601 | Trungnxqe170172@fpt.edu.cn | 2 | SE17B02 |
| Screen shot: https://www.tinkercad.com/things/jc1zb5mJ3y2-brave-leelo/editel?sharecode=CyC59Kb8jsjHsDneupmbkvPrirXQVt1fmdUEIUf9SPg |  |  |  |  |